# Matek H743 Flight Controller Parameter Configuration Summary

Configuration export: 6 October 2026  |  ArduCopter 4.6.2  |  Vehicle system ID 1

This page summarizes the 1,283-parameter export recorded from the physical Matek H743-SLIM V3 quadcopter. It documents the configuration used during bench verification and highlights settings requiring review before physical flight testing.

| Area | Recorded parameter values | Meaning for the system |
| --- | --- | --- |
| Airframe | FRAME_CLASS 1; FRAME_TYPE 12 | Quadcopter using the Betaflight X motor arrangement. |
| Motor output | MOT_PWM_TYPE 6 | DShot600 digital signalling to the electronic speed controllers. |
| Vehicle identity | SYSID_THISMAV 1 | The flight controller is addressed as vehicle 1 in MAVLink. |
| Position control | PSC_POSXY_P 1.0; PSC_VELXY_P 1.5 | Horizontal position and velocity controller gains used by ArduPilot. |
| Navigation limits | WPNAV_SPEED 1600; WPNAV_ACCEL 250 | Maximum waypoint speed 16 m/s and acceleration 2.5 m/s^2. |
| Recovery and landing | RTL_ALT 5000; LAND_SPEED 50 | Return altitude 50 m and final landing descent speed 0.5 m/s. |
| Battery monitoring | BATT_MONITOR 4 | Analog voltage and current monitoring is configured. |
| Battery failsafe | BATT_LOW_VOLT 0; BATT_CRT_VOLT 0; actions 0 | Low and critical thresholds/actions were not configured in this export. |
| Control failsafes | FS_THR_ENABLE 1; FS_GCS_ENABLE 0 | Throttle failsafe enabled; ground-station-loss failsafe disabled. |
| Arming and fence | ARMING_CHECK 112; FENCE_ENABLE 0 | A partial arming-check mask is used; geofence disabled. Both require safety review. |
| MAVLink | SERIAL0_PROTOCOL 2; SERIAL1_PROTOCOL 2; BAUD 57 | MAVLink 2 enabled on USB and Telemetry 1; Telemetry 1 set to 57,600 baud. |
| CAN | CAN_D1_PROTOCOL 1; CAN_P1_DRIVER 0 | DroneCAN protocol selected, but the physical CAN driver was disabled. |
| Flight logging | LOG_BACKEND_TYPE 1; LOG_DISARMED 0 | Onboard file logging selected; recording starts only after arming. |

Bench verification status

The controller connected successfully through USB-C MAVLink. The gyroscope, accelerometer, compass, barometer and GPS reported healthy, and the GPS obtained a 3D fix with 23 satellites. The vehicle remained disarmed. Pre-arm messages reported that accelerometer calibration was required and that no RC input was detected.

Interpretation. The export confirms that the flight controller and major interfaces were configured and readable. It does not establish flight readiness. Calibration, RC operation, battery and GCS failsafes, geofencing, logging and a supervised single-aircraft flight test should be completed before swarm flight testing.